d ## Buildings Department Lands Department Planning Department ### Joint Practice Note No. 4

###### Development Control Parameters

### Introduction

This Joint Practice Note promulgates the practices adopted by the Buildings Department (BD), Lands Department (LandsD) and Planning Department (PlanD) in respect of some development control parameters that are commonly encountered in the development process.

Practices amongst BD, LandsD and PlanD

2.			Those development control parameters on which the three departments have broadly adopted similar practices in the past, subject to certain qualifications, are set out in the attached Appendix.

3.			Notwithstanding the above, developers and their professional advisers should be aware that BD and PlanD are acting in accordance with their specific statutory powers and regulations, whereas in the case of LandsD, the actual wording adopted for a specific item of the development control parameters in the lease or grant conditions for the specific lot concerned will still need to be subject to legal interpretation to ascertain the precise effect of the provisions in the lease or grant conditions in the context of the development proposed.

4.			Furthermore, it must be noted that LandsD acts in its role on behalf of Government as landlord.  It reserves the right to interpret the legal meaning and effect of any particular lease or grant condition for a specific lot in relation to any of these development control parameters in accordance with the private law principles of landlord and tenant and any other relevant legal principles. It is not bound by the interpretations adopted by BD or PlanD in the context of their relevant statutes in any way.

5. 			It must be emphasized that the above elaboration should not be regarded as exhaustive.  Owing to the variance of development parameters between individual leases and properties, each case will be considered on its own merit by LandsD.

| (C K AU) | (Miss Annie TAM) | (Mrs. Ava S.Y. NG) |
| --- | --- | --- |
| Building Authority | Director of Lands | Director of Planning |

### Ref.:	BD GR/1-125/6/1/0 (II)

LD 1/3/1020/96 XX

### TPB/S/OTH/13

This Issued : 25 November 2008

Index under : 	Alignment of Development Control Parameters

## Appendix

###### Development Control Parameters on which BD, LandsD and PlanD Adopted Similar Practices Broadly

| # Items | BD | LandsD1 | PlanD |
| --- | --- | --- | --- |
| ### Gross Floor Area (GFA) calculation |  |  |  |
| Public transport terminus | All PTT should be accountable for GFA unless otherwise provided for in the relevant statutory town plan or covered by planning approval. | All PTT should be accountable for GFA unless otherwise provided for in the lease. | All PTT should be accountable for GFA unless otherwise provided for in the relevant statutory town plan or covered by planning approval. |
| Government Accommodation (to be handed over to Government) other than public transport terminus | Accountable for GFA. | Accountable for GFA unless exemption is specifically provided for in the lease. | Accountable for GFA unless otherwise provided for in the relevant statutory town plan or covered by planning approval. |
| Covered public carpark | Accountable for GFA. | Accountable for GFA. | Accountable for GFA. |
| ### Covered private carparking space, loading and unloading bay | Not accountable for GFA subject to:  design to the satisfaction of the BA; and  provision not excessive with reference to HKPSG or requirements stipulated by the Commissioner for Transport. | Not accountable for GFA subject to:  provision not exceeding the minimum requirements stipulated in lease conditions; and  design to the satisfaction of the Director of Lands/Commissioner for Transport. | Ancillary facilities in line with HKPSG or requirements stipulated by the Commissioner for Transport will be exempted. |
| ### Lobbies in carparking floors | Not accountable for GFA if the whole part of the floor served by the lobby concerned does not have accountable GFA, and the lobby is not provided at the level(s) of the street(s) to serve as main access to the development (PNAP 13 refers). | Follow BD's practice and in case of doubt, reference should be made to LandsD Practice Note No. APSRSE 1/98. | Follows BD’s practice. |
| Covered walkways for residential developments | Open-sided covered walkways may be exempted from GFA calculation subject to compliance with the criteria laid down in PNAP 116. | Follow BD's practice and in case of doubt, reference should be made to LandsD Practice Note No. APSRSE 1/98. | Follows BD’s practice. |
| Recreational facilities | May be exempted from GFA calculation subject to compliance with the criteria laid down in PNAP 229. | May be exempted from GFA calculation subject to compliance with the criteria laid down in LandsD Practice Note No. 4/2000. | May be exempted from GFA calculation if provided for in the relevant statutory town plan or covered by planning approval. On the treatment of recreational facilities, generally follows BD's practice. |
| Open flat roofs | Not accountable for GFA. | Not accountable for GFA in situations of reasonable building set-backs. | Not accountable for GFA. |

The question of GFA accountability is only relevant for lease conditions which contain a reference to maximum permissible GFA.