| ## Buildings Department | Practice Note for Authorized Persons and Registered Structural Engineers | 79 |
| --- | --- | --- |

Computer Programs for Use

in Structural and Geotechnical Design

An authorized person (AP), a registered structural engineer (RSE) or a registered geotechnical engineer (RGE) may use computer programs to produce calculations to support his design or to produce designs for submissions to the Building Authority (BA).  Where he intends to do so, he should ensure that the program from which such calculations or designs are produced is acceptable to the BA before submitting plans for approval under the Buildings Ordinance.

Prior Acceptance of Computer Program

2.		An application for prior acceptance of structural computer program and geotechnical computer program should be made by a RSE and a RGE respectively.  A computer program accepted by the Buildings Department (BD) will be assigned a BD reference number (prefixed ‘S’ for structural and ‘G’ for geotechnical), which should be quoted in submissions.

3.		Application for prior acceptance of a computer program (including the renewal of the prior acceptance) should be made using the standard pro forma in Appendix A.  The completed pro forma should be accompanied by relevant documentation of the program including a user’s manual giving the following information:

- mathematical modelling of the analysis and/or design;

- flow charts showing the analysis and/or design procedure;

- Building Regulations, codes of practice or other requirements which the analysis or design produced by the program will comply with;

- theory of analysis and mathematical basis of the computational algorithm which are used in the program including any references;

- limitations and applicability of the program;

- structure and method of data input and output;

- /(g) …..

- interpretation of results; and

- examples which are sufficient to illustrate the operation of every part of the program at all representative input data values and verified by the RSE or the RGE, as appropriate to the type of computer program.

Additional test examples may be required in order that the accuracy of the computer program can be fully demonstrated.

Use of computer Program in Submission of Plans for Approval

4.		When using the program to support structural or geotechnical design, the AP or RSE or RGE is required to provide the information in the first part of the structural calculations as stipulated in paragraph 9(b) of PNAP 121.

- 5.		For structural computer programs, the RSE should be responsible for the program execution. Similarly, a RGE should be responsible for the program execution for geotechnical computer programs.  The AP/RSE/RGE is also required to complete the standard pro forma in Appendix B and/or C and include it in the submission.

6.		When a submission is supported by a computer program which does not bear a valid prior acceptance, the BA may require all information listed in paragraph 3 to be included in the submission for consideration.

# Renewal of Prior Acceptance and Changes to Pre-Accepted Programs

7.		The prior acceptance of any computer program will be valid for a period of up to three years.  In view of advances in science and technology and periodic revision to Codes of Practice, prior acceptance of any computer program will be considered to have expired on a date three years from the date of last acceptance.

8.		Application for renewal of the prior acceptance may be made upon expiry, but not earlier than four months from the expiry date.  Such applications will be examined by BD in the light of the requirements set out in this practice note.

9.		If changes are proposed to an accepted program, details of the amendments and the updated user manual (if applicable) should be submitted to BD for acceptance before using the revised program.

/10. …..

10.		Lists of currently accepted structural and geotechnical computer programs, and program limitations are available for inspection at the general registries of BD on 13/F, 14/F & 15/F of Pioneer Centre, 750 Nathan Road, Mong Kok, Kowloon or through the website of the Buildings Department (www.bd.gov.hk)

( H W CHEUNG )

Building Authority

- Ref.: BD GR/OA/71/1

- First issue  December 1981

- Last revision  September 2002

- This revision  December 2005 (AD/Sup)  (Paras. 1, 2, 3, 4 & 5 amended to include

- RGE)

Index under: Computer Programs