| Buildings Department |  | Practice Note for Authorised Persons and Registered Structural Engineers | 252 |
| --- | --- | --- | --- |

Management Framework for

Disposal of Dredged/Excavated Sediment

Introduction

This practice note is issued to provide information on the management framework for dredged/excavated sediment disposal under the Dumping at Sea Ordinance, Cap. 466 (DASO).  It classifies sediment into three categories based on their contaminant levels, outlines the procedures to be followed in assessing sediment quality and explains the marine disposal arrangement for the different sediment categories.

2.		This practice note applies to all private projects which involve the marine disposal of dredged/excavated sediment and whose dredging/excavation work will commence on or after 1 January 2002.  Projects for which the dredging/excavation work has already commenced, or will commence before 1 January 2002 may continue to follow the previous arrangement set out in Practice Note for Authorized Persons and Registered Structural Engineers No. 155. Example of projects include reclamation, construction of marinas and deep excavation for basement construction where the excavated sediment is of such quantity that marine disposal has to be adopted.

Responsibilities of Environmental Protection Department (EPD) and Marine Fill Committee (MFC)

3.		The Director of Environmental Protection (DEP) is the authority to issue dumping permits for marine disposal as required under DASO.  The Marine Fill Committee (MFC), of which the Director of Civil Engineering (DCE) is the Chairman, is responsible for establishing and managing suitable facilities for the disposal of dredged/excavated sediment including advising DEP on the allocation of disposal space at these facilities.

4.		DEP controls dumping at sea by means of permits which are issued to contractors or other parties responsible for the disposal of dredged/excavated sediment.  Permits are valid for specific periods of time, for specific quantities and types of material carried by specific vessels, for a specific/designated place of disposal, and are subject to certain conditions.  Failure to comply with permit conditions will render the permit holder and/or responsible parties liable to legal action, and for repeated offences to refusal of future permits. This practice note provides guidance for the authorized persons (AP)/registered structural engineers (RSE) to obtain information on the sediment which can then be used for supporting the permit application by the contractors and/or other parties responsible for the disposal.

/Classification …..

Classification of Sediment

5.		DEP, as the Authority under the DASO, will classify sediments based on their contaminant levels with reference to the Chemical Exceedance Levels (CEL) laid down in Appendix A.  There are 3 categories of sediment :

Category L :	Sediment with all contaminant levels not exceeding the Lower Chemical  Exceedance Level (LCEL).  The material must be dredged, transported and disposed of in a manner which minimizes the loss of contaminants either into solution or by resuspension.

Category M :	Sediment with any one or more contaminant levels exceeding the Lower Chemical Exceedance Level (LCEL) and none exceeding the Upper Chemical Exceedance Level (UCEL).  The material must be dredged and transported with care, and must be effectively isolated from the environment upon final disposal unless appropriate biological tests demonstrate that the material will not adversely affect the marine environment.

Category H :	Sediment with any one or more contaminant levels exceeding the Upper Chemical Exceedance Level (UCEL).  The material must  be dredged and transported with great care, and must be effectively isolated from the environment upon final disposal e.g. by depositing it in a pit on the seabed and covering it up with clean material.

Determination of Sediment Quality

6.		The AP/RSE will, in consultation with DEP, conduct an initial assessment on whether the existing data can conclusively demonstrate that the sediment is suitable for open sea disposal.  If no such conclusion can be made, the AP/RSE must submit proposals for sampling and chemical testing of the sediment to DEP for approval and copy to the Secretary of the MFC in the Civil Engineering Department together with details of the anticipated disposal requirements.

7.		Upon completion of the sampling and chemical testing, the AP/RSE shall submit a Preliminary Sediment Quality Report (PSQR) to DEP and copy to the Secretary of MFC.  This report shall include the sampling details, the chemical testing results, quality control records, proposed classification and delineation of sediment according to paragraph 5 above, and the information and/or records as specified by DEP in his approval of sediment sampling and testing plan.

/8. …...

8.		If Category M sediment and/or Category H sediment are found in the sediment, the AP/RSE will be required to subject samples of Category M and/or certain Category H sediment for biological testing and submit a test proposal to DEP for approval which should also be copied to the Secretary of MFC.  Upon completion of the biological testing, the AP/RSE shall submit a formal Sediment Quality Report (SQR) for both the chemical and biological tests for DEP’s approval.  For projects involving sediment dredging, the SQR must be completed and submitted to DEP at least 3 months prior to the dredging contract being tendered or at least 2 months prior to the works order for maintenance dredging being issued.  In cases where biological testing is not required, subject to the approval of DEP, the PSQR will be deemed to be the formal SQR.

9.		Since the seabed or riverbed conditions may have changed significantly if the time gap between sampling and testing and the actual dredging is too long, the AP/RSE has to ensure that the data in the SQR is still reliable at the time of applying for allocation of sediment disposal space from MFC and at the time of applying for a dumping permit under the DASO.  At the time of approval of the sampling and chemical testing proposal for the sediment, DEP will specify the period beyond which the reliability of the data in the SQR, to which the testing proposal relates, has to be reviewed.  This period starts to count on the actual date of commencement of sampling and will be not less than three years.  The same period will apply to any subsequent reviews if needed.

10.		The AP/RSE may however initiate a review, before or after the expiry of the period specified, to meet the needs of MFC allocation and issue of the dumping permit. In such case, the AP/RSE shall obtain DEP’s prior agreement on the review methodology and sampling locations.  The review will normally require sampling at 5-10% of the previously approved sampling locations and conducting of chemical tests for comparison with previous chemical test results.  If the results of the review indicate that the quality of the sediment has changed significantly to the extent that the previously approved disposal arrangement may require modification, further sampling and testing to update the data of the SQR will be required.

11.		Guidelines on the initial data assessment, the sampling and testing procedures, the biological test criteria, and the submission requirements are set out in Appendix B.  The sampling and testing procedures are critical to the accurate evaluation of the sediment contamination, and close supervision by the AP/RSE is therefore necessary.  The AP/RSE  should ensure that all the sampling and testing requirements are fully complied with to avoid subsequent delay in the permit application.

12.		In cases of emergency dredging for safety reasons, and for small scale dredging works of a maintenance nature and involving dredging volumes of less than 5,000 m3 in situ, DEP may waive the sediment testing procedures as mentioned in paragraphs 6 to 8. In such cases, previously  obtained data or known history of the sediment in the concerned area or in the vicinity should be submitted to DEP for consideration of the most appropriate arrangements for handling these materials.

/Allocation …..

Allocation of Sediment Disposal Site

13.		The allocation of sediment disposal space will not be considered until the need for removal of the sediment has first been satisfactorily demonstrated. The rationale for sediment removal must therefore be provided to the Secretary of the MFC for agreement, as early as possible, preferably at the Environmental Impact Assessment Stage, if one is conducted. Volumes of Category L sediment below 50,000 m3 are exempted from this requirement.

14.		After the volume of sediment to be removed has been agreed, the Secretariat of the MFC will provide an allocation of disposal space at an appropriate marine site or sites.  For projects with disposal requirements of less than 50,000 m3 Category L sediment, the allocation of disposal space has been delegated to DEP.

15.		On the basis of the chemical and biological test results, MFC will determine the most appropriate open sea or confined marine disposal site in accordance with the following table and as depicted in Appendix C.

Determination of sediment disposal option

| Sediment Category | Disposal Option |
| --- | --- |
| Category L sediment | Type 1 - Open sea disposal |
| Category M sediment Passing biological test | Type 1 - Open sea disposal (dedicated sites) |
| Category M sediment Failing biological test | Type 2 - confined marine disposal |
| Category H sediment Biological test not required / Passing biological (dilution) test* | Type 2 – confined marine disposal |
| Category H sediment Failing biological (dilution) test | Type 3 – special treatment / disposal (site and project specific) |

* See Appendix B section 3(a)

16.		If any seriously contaminated sediment (Category H sediment failing the biological dilution test) has been identified, it will be the responsibility of the AP/RSE to identify and agree in consultation with DEP the most appropriate treatment and/or disposal arrangement.  Such a proposal is likely to be very site and project specific and therefore cannot be prescribed. This will not preclude treatment of seriously contaminated material to render it suitable for confined marine disposal.

/17. …..

17.		For sediment requiring Type 2 or Type 3 disposal option, contract documents shall be written to exactly reflect the allocation decision of MFC and DEP.  At present, East Sha Chau mud pits are designated for confined marine disposal.

18.		Open sea disposal sites are the South Cheung Chau spoil disposal area and the East Ninepins spoil disposal ground. The empty marine borrow pits are the dedicated open sea disposal sites. Most disposal sites are multi-user facilities and as a consequence their management involves a flexibility to accommodate varying and unpredictable circumstances.  Contract documents must therefore be written so as to allow the same degree of flexibility should it be necessary to divert from one disposal site to another during the construction period of a contract.

19.		The allocation of disposal space will normally carry a requirement for the arrangement for chemical analysis of the sediment sampled from 5% of the vessels en-route to the disposal site.  For Category M and certain Category H sediment, the chemical tests will be augmented by biological tests.  Vessel sampling will normally entail compositing five samples from the vessel and undertaking laboratory tests on the composite sample.  All marine disposal sites will be monitored under the general direction of the Civil Engineering Department.  However, exceptionally large allocations might require some additional disposal site monitoring.  These will be stipulated at the time of allocation.

20.		Trailer suction hopper dredgers disposing sediment at East Sha Chau must use a down-a-pipe disposal method, the design of which must be approved in advance by DCE.  Equipment for such disposal must be provided by the dredging contractor.

21.		For private projects allocated with marine disposal space, there will be a charge per cubic metre as measured in situ at the dredging site and as certified by the AP/RSE.

22.		The MFC Secretariat maintains a database on fill requirements and surplus materials of civil engineering projects in Hong Kong to facilitate the planning and management of fill and disposal resources.  The AP/RSE of private developments shall ensure that details of any fill requirements or disposal requirements, and the subsequent updates, are sent to the MFC Secretariat using the form at Appendix D, preferably via electronic transfer, copied to BD.

Application for Marine Dumping Permit

23.		The contractor who will be undertaking the works must make a formal application to EPD for a dumping permit, and if the permit is granted, it will be the contractor's responsibility to ensure that the permit conditions are met to DEP's satisfaction.

/24. …..

24.		Any queries regarding this practice note should be directed to the MFC Secretariat (Tel. no.: 2762 5397) or the Waste and Water Management Group (WMG) of EPD (Tel. no.: 2835 1287).

( C M LEUNG )

Building Authority

Ref : BD GP/BL/P/4 (III)

First issue  May 2001 (AD/NB1, DEP)

Index under : 	Disposal of Dredged/Excavated Sediment

Dredged/Excavated Sediment - Disposal of

Appendix A

(PNAP 252)

## Sediment Quality Criteria for the Classification of Sediment

| Contaminants | Lower Chemical Exceedance Level (LCEL) | Upper Chemical Exceedance Level (UCEL) |
| --- | --- | --- |
| Metals (mg/kg dry wt.) |  |  |
| Cadmium (Cd) | 1.5 | 4 |
| Chromium (Cr) | 80 | 160 |
| Copper (Cu) | 65 | 110 |
| Mercury (Hg) | 0.5 | 1 |
| Nickel (Ni)* | 40 | 40 |
| Lead (Pb) | 75 | 110 |
| Silver (Ag) | 1 | 2 |
| Zinc (Zn) | 200 | 270 |
| Metalloid (mg/kg dry wt.) |  |  |
| Arsenic (As) | 12 | 42 |
| Organic-PAHs (g/kg dry wt.) |  |  |
| Low Molecular Weight PAHs | 550 | 3160 |
| High Molecular Weight PAHs | 1700 | 9600 |
| Organic-non-PAHs (g/kg dry wt.) |  |  |
| Total PCBs | 23 | 180 |
| Organometallics (g TBT/L in Interstitial water) |  |  |
| Tributyltin* | 0.15 | 0.15 |

*	The contaminant level is considered to have exceeded the UCEL if it is greater than the value shown.

(5/2001)

### Appendix B

(PNAP 252)

GUIDELINES  FOR SEDIMENT ASSESSMENT

The purpose of these guidelines is to set out the requirements for assessing, sampling, testing and categorising the sediment.  All project departments or the AP/RSE must comply with these requirements when notifying Director of Environmental Protection (DEP) and Marine Fill Committee (MFC) of an intention to dredge/excavate and dispose of the sediment.

The sampling and testing procedures and the subsequent submission of a Sediment Quality Report normally require 8 months to complete.

1.	TIER I	-	REVIEW  OF  EXISTING  INFORMATION  FOR SITE CONTAMINATION  ASSESSMENT

The purpose of Tier I screening is to review available information to determine whether the sediment belongs to Category L material for open sea disposal.  If the AP/RSE considers that there is insufficient information to  make such conclusion, the AP/RSE may proceed directly to Tier II screening.

(a)	Submission requirements

The AP/RSE shall submit a formal proposal to DEP and copy to the Secretary of MFC in the Civil Engineering Department and Director of Buildings. The proposal should contain the following information :

(i)	project name;

(ii)	plan showing detailed location and boundary of the dredging /excavation site;

(iii)	estimated volume of dredged/excavated sediment requiring disposal;

(iv)	timetable for dredging/excavation operation and the corresponding disposal space required;

(v)	previous dredging/excavation history of the site;

(vi)	previous use of the site; and

- other available site specific information (e.g., sediment grain size, total organic carbon (TOC), geotechnical data, previous testing results).

(b)	Necessity to proceed to  Tier II - chemical screening

DEP will examine the submission and advise whether:

(i)	the information is sufficient to conclude that the sediment is suitable for open sea disposal and the submission can be accepted as a formal Sediment Quality Report;  or

(ii)	Tier II - chemical screening is required.

2.	TIER II	-	CHEMICAL SCREENING

Tier II screening is designed to categorise the sediment based on its chemical contaminant levels, and to determine whether the sediment is suitable for open sea disposal without further testing.

(a)	Submission requirements

The AP/RSE shall submit for approval a test proposal to DEP and copy to the Secretary of MFC.  The proposal should include the following:

(i)	project name;

(ii)	plan showing detailed location and boundary of the dredging/excavation site;

(iii)	estimated volume of dredged/excavated sediment requiring disposal;

(iv)	the anticipated timetable for taking the sample, carrying out the tests, and producing the Sediment Quality Report for chemical & biological screening;

(v)	a plan showing the area to be dredged, the locations to be sampled and their Hong Kong metric grid coordinates;

In general, the following sampling arrangement should be adopted:

| #### Expected contamination level | Recommended Sampling Arrangement |
| --- | --- |
| Low | 200 x 200 m grid, surface sample only |
| High | 100 x 100 m grid, vertical profile of samples |
| Very high (e.g. near outfalls, or nullahs) | 50 x 50 m grid, vertical profile of samples |

When biological screening is anticipated, samples of reference sediment should also be taken.  Reference sediment required for the test should be collected from a location within the general vicinity of the intended disposal site and should be selected by the proponent and agreed by DEP.  It should be outside the influence of previous disposal operations but close enough to reflect similar natural environmental characteristics (e.g. grain size, TOC).

(vi)	a schedule of the types of samples to be taken (e.g., grab samples, gravity coring, piston samples, vibrocores, etc.) with their locations and depths;

Where vertical profiles of samples are to be taken, samples should be continuous, and the top level of the sub-samples should be the seabed, 0.9m down, 1.9m down, 2.9m down, and then every 3m to the bottom of the dredged layers.

The size of samples collected should be adequate for this tier of chemical testing as well as the next tier of biological testing described in subsequent sections.

| Parameters to be tested | Sample size* |
| --- | --- |
| Metals and metalloid | 0.5 litre |
| Organic | 0.5 litre |
| Biological response | 6 litres |

*	Quantity to be confirmed by testing laboratory.  The quantity of reference sediment to be collected needs to be separately worked out for each case, especially if biological dilution tests are anticipated.

(vii)	a schedule of tests to be carried out on the samples.

Unless otherwise specified, all samples shall be tested for all the contaminants (except Tributytin (TBT)) stated in Table 1 - Analytical Methodology at the end of this Appendix plus other ancillary parameters such as moisture content, grain size, TOC, ammonia and salinity of porewater.  Analysis for other contaminants, such as TBT, Dichloro-diphenyl-trichloroethane (DDT), other organo-chlorine compounds, and other hazardous chemicals which arise from specific industrial discharge or spillage, may also be required by DEP in areas where contamination by such compounds is suspected.

(b)	Sampling and testing requirements

(i)	Sampling practice and sample storage

All sampling bottles should be labelled with the station number, sample length, diameter and depth, together with a full description of the sample.  The recommended type of sampling bottles and pretreatment method are:

| Parameters To be tested | Sampling bottle | Pretreatment procedure# |
| --- | --- | --- |
| Metals and metalloid | High density polyethylene bottles* | USEPA SW-846 + Chapter 3 |
| Organic | Wide mouth Borosilicate glass bottles with Teflon lined lid | USEPA SW-846 Chapter 4 |
| Biological response | Wide mouth Borosilicate glass bottles with Teflon lined lid or high density polyethylene bottles * | USEPA SW-846 Chapter 3 or Chapter 4 as appropriate. |

*	Heavy duty plastic bags may be used for the storage of sediment sample for testing metals, metalloid and biological response.

#	Other equivalent methods may be used subject to the approval of DEP.

+	Test methods for evaluating solid waste: physical/chemical methods, SW-846, 3rd edition, United States Environmental Protection Agency.

The samples should be kept at 4C in the dark and should not be frozen.  All samples should be promptly analysed.  If this is impractical, the recommended maximum holding time is:

| Sample type | Maximum holding time |
| --- | --- |
| Chemical test | 2 weeks |
| Biological test | 8 weeks |

(ii)	Analytical methodologies for chemical screening

The analytical method used for detecting each contaminant should be in accordance with the methodology described in Table 1 - Analytical Methodology at the end of this Appendix.

(c)	Quality assurance/quality control (QA/QC) requirements

All tests must be conducted by laboratories accredited by Hong Kong Laboratory Accreditation Scheme (HOKLAS) or, in case of overseas laboratories, by equivalent national accreditation for these tests.

(d)	Necessity to proceed to Tier III biological screening

There is no need to proceed to Tier III for Category L sediment.  However, the project proponent must proceed to Tier III for further analysis of Category M and certain category H sediment.  For the latter, Tier III screening is only required if one or more contaminant levels exceed 10 times the Lower Chemical Exceedence Level (LCEL).

3.	TIER III	  -	BIOLOGICAL SCREENING

The purpose of Tier III screening is to identify the most appropriate disposal option for Categories M and H sediments.

(a)	Submission requirements

The AP/RSE shall submit for approval a test proposal to DEP and copy to the Secretary of MFC.  The proposal should include the following:

(i)	the number of biological tests;

(ii)	the arrangement for preparing the composite samples; and

(iii)	the test species and test conditions.

In general, all biological tests should be conducted on composite samples.  Composite sample is prepared by mixing up to 5 samples of the same category (M or H) which are continuous in vertical or horizontal profile.

Sediment classified as Category M shall be subjected to the following three toxicity tests (to be considered as one set) on each composite sample:

a 10-day  burrowing amphipod toxicity test; and

a 20-day  burrowing polychaete toxicity test; and

a 48-96 hour larvae (bivalve or echinoderm) toxicity test.

Sediment classified as Category H and with one or more contaminant levels exceeding 10 times LCEL shall also be subjected to the above three toxicity tests but in a diluted manner (dilution test). The samples shall be prepared prior to toxicity testing as follows:

| Sediment characteristics | Preparation method |
| --- | --- |
| Category H sediment (> 10 x LCEL) | Sample to be mixed with 9 portions of reference sediment |
| Category M sediment or Category H sediment (> 10 x LCEL) suspected of ammonia contamination* | Additional set of sample (after dilution for Cat. H sediment) to be purged# for ammonia removal (for amphipod test only). |

*	Ammonia level  20 mg/l in the interstitial water.

#	Ammonia purging is conducted to reduce ammonia level by replacing two volumes of overlying water per day.

(b)	Testing requirements

The test endpoints and decision criteria are summarized in Table 2 at the end of this Appendix.  The sediment is deemed to have failed the biological test if it fails in any one of the three toxicity tests.

Only ecologically relevant species should be used for carrying out the biological screening tests.  The species to be used for each type of test can be selected from the following:

| Test Types | Species | Reference Test Conditions* |
| --- | --- | --- |
| 10-day burrowing amphipod toxicity test | Ampelisca abdita Leptocheirus plumulosus Eohaustorius estuarius | U.S.EPA(1994)/PSEP (1995) U.S.EPA(1994) U.S.EPA(1994)/PSEP (1995) |
| 20-day burrowing polychaete toxicity test | Neanthes arenaceodentata | PSEP(1995) |
| 48-96 hour larvae (bivalve or echinoderm) toxicity test | Bivalve: Mytilus spp. Crassostrea gigas Echinoderm : Dendraster excentricus Strongylocentrotus spp. | PSEP(1995) PSEP(1995) PSEP(1995) PSEP(1995) |

* U.S.EPA (U.S. Environmental Protection Agency) 1994.  Methods for assessing the toxicity of sediment-associated contaminants with estuarine and marine amphipods.  Office of Research and Development. U.S. Environmental Protection Agency, Cincinnati, OH. EPA/600/R94/025.

PSEP (Puget Sound Estuary Program) 1995. Recommended guidelines for conducting laboratory bioassays on Puget Sound sediments.

(c)	Quality assurance/quality control (QA/QC) requirements

All biological tests must be conducted by laboratory with appropriate accreditation.

The biological test shall include appropriate quality assurance/quality control such as :

(i)	Negative Control

(ii)	Positive Control

4.	REPORTING REQUIREMENT AFTER COMPLETION OF CHEMICAL & BIOLOGICAL SCREENING

(a)	Submission requirements

Upon completion of each stage of screening (chemical and biological), the AP/RSE shall submit to DEP and copy to the Secretary of MFC a report on the results covering all tests conducted so far. The report should include the following information where appropriate:

(i)	plans showing the delineation of each of the 3 categories of dredged/excavated material and the corresponding types of disposal required based on the chemical and biological screening results, and

(ii)	the following information:

- Name and location of the testing laboratory

- Location of samples and source of reference sediments, method of collection, handling, preservation and storage, dates and times of sample collection and receipt at the testing laboratory

- (For chemical screening)

- Dates of analysis

- Analytical methods and detection limits

- Tabulated sample results with units, including reporting basis (e.g., wet, dry, TOC normalized) and QA/QC results

- Explanations for all departures from the standard protocols and discussion of possible effects on the data

- (For biological screening)

- Test species information such as the source, size, history and age of test organisms

- Source of control seawater and control sediment used, including any pretreatment

- Preparation procedures for test sediment sample and test organisms

- Test conditions for each test including any deviation from standard procedures and discussion of possible effects on the data

- Water quality measurement during testing

- QA/QC results

- Effect measurements, end point results and their statistical significance

Table 1 - Analytical Methodology

| Parameters | Preparation Method US EPA Method | Determination Method US EPA Method | Reporting Limit |
| --- | --- | --- | --- |
| Metals (mg/kg dry wt.) |  |  |  |
| Cadmium (Cd) | 3050B | 6020 or 7000A or 7131A | 0.2 |
| Chromium (Cr) | 3050B | 6010A or 7000A or 7190 | 8 |
| Copper (Cu) | 3050B | 6010A or 7000A or 7210 | 7 |
| Mercury (Hg) | 7471A | 7471A | 0.05 |
| Nickel (Ni) | 3050B | 6010A or 7000A or 7520 | 4 |
| Lead (Pb) | 3050B | 6010A or 7000A or 7420 | 8 |
| Silver (Ag) | 3050B | 6020 or 7000A or 7761 | 0.1 |
| Zinc (Zn) | 3050B | 6010A or 7000A or 7950 | 20 |
| Metalloid (mg/kg dry wt.) |  |  |  |
| Arsenic (As) | 3050B | 6020 or 7000A or 7061A | 1 |
| Organic-PAHs (g/kg dry wt.) |  |  |  |
| Low Molecular Weight PAHs+ | 3540C and 3630C | 8270B or 8270C | 55 |
| High Molecular Weight PAHs++ | 3540C and 3630C | 8270B or 8270C | 170 |
| Organic-non-PAHs (g/kg dry wt.) |  |  |  |
| Total PCBs | 3540C and 3665A | 8082 | 3 |
| Organometallics (g TBT/L in interstitial water) |  |  |  |
| Tributyltin | Krone et al. (1989)* - GC/MS | Krone et al. (1989) - GC/MS | 0.015 |

Footnote:	(i)	The reporting limits shown in this table are the most stringent limits which will be specified by DEP.  AP/RSE should consult DEP on the required limits in the preparation of proposals for sampling and chemical testing of the sediment.

(ii)	Any methodology for which the laboratory is accredited that will produce equivalent or better results/reporting limits as required may be used subject to approval by DEP.

+	Low molecular weight PAHs include acenaphthene, acenaphthylene, anthracene, fluorene, naphthalene, and phenanthrene

++	High molecular weight PAHs include benzo[a]anthracene, benzo[a]pyrene, chrysene, dibenzo[a,h]anthracene, fluoranthene, pyrene, benzo[b]fluoranthene, benzo[k]fluoranthene, indeno[1,2,3-c,d]pyrene and benzo[g,h,i]perylene

*	Krone et al. (1989), A method for analysis of butyltin species and measurement of butyltins in sediment and English Sole livers from Puget Sound, Marine Environmental Research 27 (1989) 1-18.  Interstitial water to be obtained by centrifuging the sediment and collecting the overlying water.

Table 2	-	Test endpoints and decision criteria for Tier III biological screening

| Toxicity test | Endpoints measured | Failure criteria |
| --- | --- | --- |
| 10-day amphipod | Survival | Mean survival in test sediment is significantly different (p  0.05)1 from mean survival in reference sediment and mean survival in test sediment < 80% of mean survival in reference sediment. |
| 20-day polychaete worm | Dry Weight2 | Mean dry weight in test sediment is significantly different (p  0.05)1 from mean dry weight in reference sediment and mean dry weight in test sediment < 90% of mean dry weight in reference sediment. |
| 48-96 hour larvae (bivalve or echinoderm) | Normality Survival3 | Mean normality survival in test sediment is significantly different (p  0.05)1 from mean normality survival in reference sediment and mean normality survival in test sediment < 80% of mean normality survival in reference sediment. |

1	Statistically significant differences should be determined using appropriate two-sample comparisons (e.g., t-tests) at a probability of  p0.05.

2	Dry weight means total dry weight after deducting dead and missing worms.

3	Normality survival integrates the normality and survival end points, and measures survival of only the normal larvae relative to the starting number.

(5/2001)

Management Framework for Dredged/Excavated ediment

Tier I

Desk Top Study

of Available Data

Tier II

Chemical Screening

Insufficient Data or Data Indicates

Potential Contamination

Data Indicates

Little or No

Contamination

Category M

Material

>Lower &



Upper Chemical

Exceedance Level

Category H

Material

>Upper Chemical

Exceedance Level

Category L

Material

<Lower Chemical

Exceedance Level

>

10

x Lower Chemical

Exceedance Level

Tier III

Biological Screening

(Dilution Test)

Tier III

Biological Screening

Type 1 -

Open Sea

Disposal

Type 1 -

*Open Sea Disposal

(Dedicated Sites)

Type 2 -

Confined

Marine Disposal

Type 3 -

Special Treatment/

Disposal

Yes

No

Fail

Pass

Fail

Pass

*

Dedicated Sites will be monitored to confirm that there is no adverse impact.

Sediment to be Disposed of

Appendix C

(PNAP 252)

FILL REQUIREMENTS AND SURPLUSES DATA FORM

(available at http://www.info.gov.hk/ced/eng/services/fm/imsweb/pdf/blank.pdf)

| PROJECT NAME |  |  |  |  |  | CLIENT |  |  | GOVT./PRIV. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CONTRACT NAME |  |  |  |  |  | DEVELOPER |  |  |  |  |
| CO-ORDINATES | E |  |  | N |  | CONTRACT NO. |  |  |  |  |
| PWP CATEGORY |  |  |  |  |  | CONTRACTOR |  |  |  |  |
| GOVT. REP. (Pre-contract) |  |  |  |  |  | GOVT.REP. (Contract) |  |  |  |  |
| CONSULTANT (Pre-contract) |  |  |  |  |  | CONSULTANT (Contract) |  |  |  |  |
| KEY DATES | Tender Documents |  | / / | Call Tenders | / / | Contract start | / / |  | Contract completion | / / |

|  | ANNUAL FILL REQUIREMENTS |  |  |  |  |  |  |  |  | ANNUAL SURPLUSES |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | GENERAL FILL |  |  |  | SPECIAL ROCK FILL |  |  |  |  |  |  |  |  |  |
|  | Marine Sand | Land Source | Public Fill | PFA | Filter | <225 mm crush/screen | Pell Mell | Armour |  | Sediment * |  | Rock | Soft Material | Other Material |
|  |  |  |  |  |  |  |  | 500-2000kg | >2000kg | Contam. | Uncontam. |  |  |  |
| Top Size | N/A |  | N/A | N/A |  |  |  |  |  | N/A | N/A |  | N/A |  |
| Bottom Size | N/A | N/A | N/A | N/A |  |  |  |  |  | N/A | N/A |  | N/A | N/A |
| Start Date | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / |
| Finish Date | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / | / / |
| Calendar Year | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. | Mil.cu.m. |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FILL SOURCE/ DISPOSAL SITE (If Finalised) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Allocated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| REMARKS |  |  |  |  |  |  |  |  |  | Filling / Excavation Works Substantially Completed |  |  |  | Yes / No |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | For MFC Secretariat Use Only | PID = CID = |  |  |  |
| FORM FILLED BY | Name |  | Office |  | Phone |  | Fax |  | E mail |  |  | Date | / / |  |

NOTE :	Annual quantities (in million cubic metres) should be entered to the nearest TWO decimal places.	Marine Fill Committee Secretariat

* Sediment Quantities to the nearest THREE decimal places.	Geotechnical Engineering Office, CED.