Practice Note for Authorized Persons

and Registered Structural Engineers

Buildings Department

212

Fire  Safety  (Commercial  Premises)  Ordinance

Introduction

The  Fire  Safety  (Commercial  Premises)  Ordinance  (“the  Ordinance”)  has come into  operation  since  2  May  1997.  The purpose of the Ordinance is to provide better protection from the risk of fire for occupants and users of, and visitors to, certain kinds of prescribed  commercial premises which have a relatively high customer density and a frequent turnover of customers.

Prescribed Commercial Premises

2.		A building or part of a building is prescribed commercial premises under the scope of the Ordinance if :-

(a)	the building or part is used, or is proposed to be used, for carrying on any of the following commercial activities :-

(i)	banking (other than merchant banking);

(ii)	conduct of off-course betting;

(iii)	conduct of a jewelry or goldsmith’s business on premises that have a security area;

(iv)	use as a supermarket, hypermarket or department store;

(v)	use as a shopping arcade;

and

(b)	the total floor area of the building or part exceeds 230m2.

3.		For the purpose of calculating the floor area of the prescribed commercial premises in paragraph 2(b),  reference should be made to Sections 3(3) to 3(6) of the Ordinance.  Appendix  A  illustrates some  examples  on demarcation  of  total  floor  area  of  shopping  arcades.  If difficulty is encountered in ascertaining whether certain commercial premises falls within the purview of the Ordinance, AP/RSE may approach the Buildings Department for clarification.

- - - - - - -

Fire Safety Measures

4.		An owner/occupier of the prescribed commercial premises may be required to comply with all or any of the following fire safety measures :-

/  (a)  Provision.....

-  2  -

(a)	Provision of additional fire safety installations and equipment

(i)	automatic sprinkler system;

(ii)	automatic cut-off devices for mechanical ventilating systems;

(iii)	emergency lighting;

(iv)	manual fire alarms;

(v)	portable fire extinguishers; and

(vi)	the requirements specified in the Code of Practice for Minimum Fire Service Installations and Equipment 1994.

(b)	Construction requirements

(i)	provision of adequate means of escape from the premises in the event of fire;

(ii)	provision of adequate means of access to the premises to facilitate access for firefighting and rescue; and

(iii)	provision of measures to inhibit the spread of fire and to ensure the integrity of the structure of the building where the premises are located.

Enforcement Authority

5.		For the purpose of this Ordinance :-

(a)	The Director of Fire Services is the enforcement authority for the provision of fire service installations or equipment;

(b)	The Director of Buildings is the enforcement authority for the provision of fire safety measures in the planning, design and construction of the premises.

Fire  Safety  Section

6.		A  new  Fire  Safety  Section  has been  set  up  in  the  Buildings  Department. This  section  is  headed  by  Chief  Building  Surveyor/Fire  Safety.

Fire  Safety  Requirements

7.		Under  section  5(1)  of  the  Ordinance,  the  Director  of  Buildings  may  serve  on  the  owner  of  prescribed  commercial  premises  a  fire  safety  direction  directing  him  to  comply  with  all  or  any  of  the  requirements  specified  in  the  following  codes  of  practices:-

|  | - the Code of Practice for the Provision of Means of Escape in Case of Fire 1996; |
| --- | --- |
|  | - the Code of Practice for Fire Resisting Construction 1996; and |
|  | - the Code of Practice for Means of Access for Firefighting and Rescue 1995. |

/  Under.....

-  3  -

8.		Under  section  6(1)  of  the  Ordinance,  the  Director  of  Buildings  may  also  apply  to  a  magistrate  to  make  a  fire  safety  compliance  order  directing  the  owner  to  comply  with  all  or  any  of  the  requirements  specified  in  the  fire  safety  direction.

Difficulties  in  complying  with  fire  safety  requirements

9.		Whilst it is important to provide better fire safety standards to prescribed commercial premises, the Director of Buildings recognizes  that  in  some  situations  there  may  be practical  and/or  administrative  difficulties  in  upgrading  the  fire  safety  measures  to meet  the  current  standards.   In  cases  of  genuine difficulties, the Director of Buildings  is  prepared  to  consider  alternative  remedial  proposals  based  on  fire  safety  engineering  principles, which will equally achieve the primary  objectives  of the Ordinance.  Discussions on proposed remedial options will be welcomed.

Submission  of  plans  for  approval

10.		Where  building  works  are proposed  either  to  voluntarily  comply  with  the Ordinance  or  to  satisfy  the  requirements  of  a  fire  safety  direction,  an application  for approval  under  the  Buildings  Ordinance  should  be  made  in  the  normal  manner.  For  such  applications,  three  sets  of  plans  showing  fire  service  installations  and  equipment  are required  by Fire  Services  Department  for  processing.

11.		Where  remedial  proposals for improvement works are  submitted  pursuant  to  the  requirements  of a fire safety direction,  no  fee  will  be  charged  under  Building  (Administration)  Regulation  42  if  the  proposals  are  entirely  for  improving  the existing premises  other  than  for  remedying  deviations  from  the  original  approved  plan.

(CHOI  Yu-leuk)

Building  Authority

Ref.:  BD  C&E/OA/183  (FS)

First  issued  August  1997  (AD/C)

Index  under:	Fire  Safety  (Commercial  Premises)  Ordinance

Prescribed Commercial Premises

Prescribed Commercial Activities

Fire Safety Direction