| Buildings Department | Practice Note for Authorized Persons and Registered Structural Engineers | 248 |
| --- | --- | --- |

Aluminium Windows

# Introduction

Aluminium windows have become popular in new developments and as replacement of steel windows in existing buildings.  Recent incidents of failure of aluminium windows, however, have aroused safety concern as falling of a dislodged casement from a height can be catastrophic in densely populated areas.

2.			Building (Construction) Regulation 3 stipulates that materials used in any building works or street works shall be of a suitable nature and quality for the purposes for which they are used; adequately mixed or prepared; and applied, used or fixed so as to perform adequately the functions for which they are designed.  Obviously such requirement is applicable to aluminium windows used in development projects.  This practice note outlines guidelines on the design and installation of aluminium windows and on the prevention of water seepage.

Design and Installation Requirements

3.			For large windows falling within the criteria stipulated in Practice Note for Authorized Persons and Registered Structural Engineers (PNAP) 239, their structural design should be submitted to the Building Authority (BA) for approval in accordance with the requirements laid down therein.  For other windows, Authorized Persons (AP) and Registered Structural Engineers (RSE) should ensure that they are properly designed and installed to meet the performance requirements and the safety standards in the Building (Construction) Regulations (B(C)Rs) whilst Registered Contractors should have experienced and skilled labour as well as suitable quality assurance procedures in place to ensure the proper installation of the windows.  Some relevant standards, specifications and codes of practice on materials, design and installation of aluminium windows are given in Appendix A for reference and information.  AP and RSE could also make reference to other national or international standards.

4.			Window members, transoms and mullions together with the glass panes should be of adequate size and strength taking into consideration the location, height and orientation of the windows.  Window frames should be securely and rigidly fixed in place to window openings in walls.  Where fixing lugs are adopted, they shall be of stainless steel or hot dip galvanized steel having a minimum material thickness of 1.5mm and be placed at 300mm centres maximum.  Where a spacing greater than 300mm is proposed, the AP/RSE should satisfy that the performance of the windows including structural stability and waterproofing would not be compromised.  Adequate site supervision should be provided to check that all the fixing lugs are properly fixed.

/5. …..

5.			All structural members of a window section shall have a minimum aluminium thickness of 2mm and the depth of the mullion section shall not be less than 38mm.  Any aluminium windows deviating from these requirements should be separately substantiated.  Particular attention shall be paid to the fixing details of the hinges.  All hinges and fastening mechanisms adopted in the installation shall withstand the positive and negative pressures due to the designed wind conditions when the window is closed and shall be of adequate size and strength commensurate with the size of the window.

6.		Where 4-bar hinges are adopted, reference should be made to the Guidelines on Fixing of 4-bar Hinges at Appendix B.

Water Seepage

7.		Besides safety consideration, water leakage from windows in new buildings poses undue nuisance to the occupants.  AP and RSE are reminded to bear this in mind when detailing window design and contractors are reminded that poor workmanship is a major cause of the problem.  Reference may be made to the Guidelines on Prevention of Water Seepage in New Buildings issued by the Buildings Department in March 2005.

## Water Tightness Test

8.		It is advisable to perform field water test on the installed aluminium window to ensure water tightness and quality of the completed window unit.  A recommended testing method is given in Appendix C for reference.

## General

9.		A similar practice note has been issued to Registered Contractors.

( Marco M H WU )

Building Authority

Ref. : BD GP/BORD/105

BD GP/BREG/P/38 (II)

First issue  July 2001 (AD/NB1, AD/NB2)

This revision  March 2005 (AD/NB1) –  paras 2 to 8 & Appendix B amended and Appendix A added

Index under :	Aluminium Windows

Windows

Water Seepage

### Appendix A

(PNAP 248)

#### Standards and Codes of Practice

#### On Aluminium Window in Building Works

The various standards and codes of practice on aluminium window in building works as listed below are intended to provide reference information for the purpose of specifying materials and design for aluminium windows and the lists are not exhaustive.

| BS EN ISO 1461 | : | Hot dip galvanizing |
| --- | --- | --- |
| BS 952: Part 1 | : | Glass for glazing |
| BS 1161 | : | Aluminium alloy sections for structural purposes |
| BS 1449: Part 2 | : | Stainless and heat resisting steel plate, sheet and strip |
| BS 1470 | : | Aluminium plate, sheet and strip |
| BS 1471 | : | Aluminium drawn tube |
| BS 1474 | : | Aluminium bars and extrusions |
| BS 1615 | : | Anodic oxidation coatings on aluminium and its alloys |
| BS 3111 | : | Stainless steel fasteners |
| BS 3987 | : | Anodic oxidation coatings on wrought aluminium |
| BS 4479 | : | Recommendations for the design of metal articles that are to be coated |
| BS 4873 | : | Aluminium alloy windows |
| BS 5889 | : | Sealants |
| BS 6105 | : | Corrosion-resistant stainless steel fasteners |
| BS 6262 | : | Code of Practice for glazing for buildings |
| BS 6375: Part 1 and Part 2 | : | Performance of windows |
| BS 8118 | : | Structural use of aluminium |
| CP 3012 | : | Cleaning and preparation of metal surfaces |
| PD 6484 | : | Commentary on corrosion at bimetallic contacts and its alleviation |

2.		AP and RSE may also make reference to other national or international standards and specifications.

(3/2005)