| ### Buildings Department | Practice Note for Authorized Persons and Registered Structural Engineers | 266 |
| --- | --- | --- |

## Level Difference for

## Floor Adjoining External Ground Level or Flat Roof

Regulations 35 and 49(1) of the Building (Construction) Regulations require that the level of an internal floor shall be not less than 150 mm above the level of the external ground/adjoining flat roof.  The objective of these requirements is to prevent the ingress of rainwater.

2.	These requirements can generally be catered for at the design stage by allowing sufficient difference in level between the internal floor and the external ground/adjoining flat roof.  However, there could be cases where authorized persons wish to depart from the prescribed requirements due to special circumstances of their cases or as a result of the unique nature of their designs.  In such circumstances, the Building Authority is prepared to consider granting exemption from the above regulations to allow flexibility in design if means to guard against the ingress of water to the inside of the building are provided to the satisfaction of the Building Authority.

( C M LEUNG )

Building Authority

Ref. :	BD GP/BREG/C/23 (VI)

First issue December 2001 (AD/NB1)

Index under :	External ground

Flat Roof

Roof

B(C)R 35

B(C)R 49(1)