| Buildings Department |  | Practice Note for Authorized Persons, Registered Structural Engineers and Registered Geotechnical Engineers | APP-130 |
| --- | --- | --- | --- |

Lighting and Ventilation Requirements – Performance-based Approach

The Buildings Department has completed a consultancy study to review the standards of the lighting and ventilation requirements in buildings.  Based on the findings of the consultancy study, the Building Authority (BA) is prepared to accept the following alternative performance standards on the provision of natural lighting and ventilation in habitable rooms and domestic kitchens for the purpose of Building (Planning) Regulations (B(P)Rs.) 30, 31 and 32:

a)	Natural Lighting

| Room of domestic building | Vertical Daylight Factor (VDF) (measurement taken on the centre of the window pane) |
| --- | --- |
| Habitable Room | 8% |
| Kitchen | 4% |

b)	Natural Ventilation

| Room of # domestic building | Air Change per Hour (ACH) |
| --- | --- |
| Habitable Room | 1.5 (natural means) |
| Kitchen | 1.5 (natural means) plus 5 (mechanical means) |

- The BA would accept proposals based on the above standards .  Applications to modify the prescriptive requirements set out in B(P)Rs 30, 31 & 32 will be accepted if the above performance standards are met.  The Authorized Persons (APs) may demonstrate compliance using any suitably verified and scientifically validated methods.  To assist APs in adopting the above performance-based approach in design, guidelines for a simplified assessment method and guidance notes on validation of lighting simulation are given in Appendices A and B respectively.

3.	The BA has no objection to accepting the design of a project partly based on the prescriptive requirements and partly based on the performance standards set out in paragraph 1.  For the purpose of B(P)R 31(2), the base may be measured at an angle not more than 15o from the external wall.

/4. The…..

4.	 The need for any refinement to the standards and replacement of the current prescriptive regulations would be considered when a review on the application of the new alternative performance standards has been conducted.

5.	Practice Note for Authorized Persons, Registered Structural Engineers and Registered Geotechnical Engineers APP-113 will not be applied to new building proposals or major revision proposals submitted after 15 March 2004.

6.	      In the case of wholesale conversion of existing industrial buildings or other types of buildings to office use where there are difficulties in providing the required natural lighting and ventilation due to constraints posed by the original design of the existing building, application for modification of B(P)Rs 30 and 31 will be favourably considered if adequate artificial lighting and mechanical ventilation and energy efficient design that could achieve 40% in the categories of Energy Use (EU) and Indoor Environmental Quality (IEQ) under the BEAM Plus certification with provisional assessment reports conferred by the Hong Kong Green Building Council are incorporated in the proposal.  As a condition for the granting of modification, supporting documents to demonstrate achievement of 40% in the categories of EU and IEQ in the Final Assessment of the BEAM Plus certification are required to be submitted within 3 months of BA’s letter of acknowledgement of the Form BA14 for the project.

( AU Choi-kai)

Building Authority

Ref :	BD GP/BREG/P/42

BD GP/BREG/P/18/1(E) (IV)

BD GP/BREG/P/18/1(G) (II)

This PNAP is previously known as PNAP 278

First issue  December 2003

Last revision  June 2005

This revision  September 2010  (AD/NB1) –  paras 1, 2, 4 and 5 revised; para 6 added

"Vertical Daylight Factor" means the ratio in percentage of the total amount of illuminance falling onto a vertical surface of a building to the instantaneous horizontal illuminance from a complete hemisphere of sky excluding direct sunlight.  It takes into account light coming from the sky directly and from reflected light of surrounding buildings and the ground both above and below the horizon.