| Buildings Department | Practice Note for Authorized Persons and Registered Structural Engineers | 279 |
| --- | --- | --- |

#### Kowloon-Canton Railway Corporation West Rail Protection

Railways Ordinance

#### Buildings Ordinance Scheduled Area No. 3

A new section of Kowloon-Canton Railway Corporation (KCRC) railway namely, West Rail (WR) consisting of at-grade, underground and viaduct structures is now under construction and expected to be completed in September 2003.  Authorized persons (APs) and registered structural engineers (RSEs) undertaking any construction activities in close proximity to this railway should pay special attention to the WR structures.  To safeguard the safety and stability of the WR structures, a “protection boundary” for the WR has been determined and a set of building/engineering guidelines has been produced.  The areas within the protection boundary are commonly known as the railway protection areas.  Similar to the railway protection areas for the Mass Transit Railway, these areas are also designated as Scheduled Area No. 3 in the Fifth Schedule to the Buildings Ordinance.  The WR railway protection areas in Scheduled Area No. 3 are the areas delineated on the plans numbered KCR/WR/RP100 to 120 (Rev. 1), KCR/WR/RP121 (Rev. 2) and KCR/WR/RP/122 to 145 (Rev. 1) which were signed by the Secretary for Housing, Planning and Lands on 6 June 2003 and deposited in the Land Registry.  Copies of these plans are available for inspection in the Buildings Department (BD) and the KCRC.

2.		As a general guide, the protection boundary is about 30 m outside the outer surface of the WR structures but it encompasses the whole of any lot where any part lies within the 30 m distance.  At the WR stations, the area enclosed by the boundary is more extensive.  Information on the "alignment of the railway as constructed" is available for viewing by APs and RSEs at the Building Information Centre of BD.  They may also be obtained direct from the KCRC.

--------	3.		The guidelines at Appendix A apply to all building works (including ground investigation works and underground drainage works) to be carried out in Scheduled Area No. 3 and any new railway protection areas to be included in the scheduled area.  Ground investigation works and underground drainage works in these areas are subject to additional control as stated in paragraph 9 below.

Building Works

4.		Plans submitted to BD for proposed building works within the protection boundary are circulated to the KCRC for its comment under the centralized processing arrangements.  Railways Ordinance section 27 will be invoked to require the incorporation of any necessary measures to protect the WR.

/5.….

5.		APs and RSEs are required to monitor any movement and vibration on the WR structures when required by the KCRC.  Upon request by the KCRC, APs and RSEs should submit a copy of the monitoring record to them.  APs and RSEs are required to inform the KCRC direct of the commencement of any building works within the protection boundary to enable them to plan an appropriate monitoring programme.  If any adverse situation becomes apparent, the KCRC will alert the interested parties.

6.		Any other necessary monitoring within the building site (e.g. monitoring of piezometric change) will be carried out by the AP/RSE of the site, who should maintain regular contact with the KCRC and keep each other informed of the monitoring records as necessary.

7.		The Secretary for the Environment, Transport and Works may in writing, under the provisions of Railways Ordinance section 24, authorize an employee of the KCRC to enter any building site within the protection boundary for the purpose of monitoring construction.

Building Opening adjacent to a West Rail Vent Shaft

8.		There are certain restraints on the design of and making alteration to properties in close proximity to a WR vent shaft to minimise the possibility of contamination by fire or smoke.  In this connection, APs and RSEs are advised that any opening such as an openable or fixed window, doorway, building ventilation system intake or exhaust and the like in any building shall be located not closer than 5 m to the opening of any WR vent shaft, irrespective of whether such vent shaft is free-standing or is accommodated in a building.  This distance may be reduced to 2.5 m, if the exhaust air from the WR vent shaft is directed away from and is not likely to affect the opening by natural convection.

Ground Investigation Works and Underground Drainage Works

9.		Ground investigation works within Scheduled Area No. 3 require prior approval and consent.  Plans prescribed under Building (Administration) Regulation 8(1)(l) should be submitted and the proposals should follow the guidelines at Appendix A.  Application for concurrent processing of approval and consent in respect of ground investigation works should follow PNAP 225.  Underground drainage works in Scheduled Area No. 3 are also subject to the full provisions of the Buildings Ordinance.

--------

Future Railway Route Protection

10.		At present, KCRC is undertaking several new railway projects which also consist of underground and viaduct structures.  On completion of these new railway projects, they may also be included in the Scheduled Area.

(Marco M H WU)

Building Authority

Ref. :	BD GP/BORD/67

BD R&L/29K/99 Pt III

First issue  August 2003 (AD/NB1)

Index under :	Kowloon-Canton Railway Corporation West Rail Protection

Scheduled Area No. 3

Appendix A

(PNAP 279)

Technical Notes for Guidance in Assessing

the Effects of Civil Engineering Construction / Building Development on

Kowloon-Canton Railway Corporation (KCRC)

West Rail (WR) Structures and Operations

A.	General

All proposals for new engineering works and developments (permanent or temporary) including public utilities located within the limits of the Railway Protection Areas, as shown in the WR Railway Protection Plans shall be subjected to the special scrutiny of the Government prior to giving approval to any plans and consent for commencing construction works.  Every proposal shall be assessed individually on its impact on existing railway and related structures including all plants and fixtures necessary for the safe operation of the railway and subjected to satisfactory compliance with the following Technical Requirements.

- Railway Underground Structures

Site Formation /Foundation Works

Where any proposed  site formation, or foundation, or bulk excavation works for basement is located above or adjacent to KCRC West Rail underground structures including cooling mains, the effects of such work shall not exceed the following limits:

- Vertical or horizontal pressure change caused either by dewatering, grouting, back filling, deep excavation, or loads transmitting from adjacent foundations or the combination of these works shall not exceed 20 kPa.

- Differential movement resulting from the works shall not produce accumulative distortion in any WR structure including the plinth and track in excess of 1 in 1000 in any plane or a total movement in any WR structure including the plinth or track exceeding 15mm in any plane whichever is smaller.

- The peak particle velocity induced to any railway structure as a result of permitted blasting work shall not exceed 25mm/sec.

- The peak particle velocity induced to any railway structure due to the driving or withdrawing of piles or any similar operation shall generally not exceed 15mm/sec, depending on the site conditions, and the duration and frequency of the activities.

/5. …..

- No pile, foundation of any kind, borehole or well, soil nails, horizontal drains, rock bolts/dowels or other geotechnical installations shall be driven or constructed within a distance of 3 m from the edge of the underground railway structures.

- Any part of a ground anchor, if permitted, shall be installed more than 3 m away from any part of a WR underground structure, and the centroid of the fixed length of the anchor shall be located away from any tunnel installation at more than twice the fixed length.

Ground Investigation Works

When ground investigation is proposed near WR’s structures, a proposal with the following details and requirements shall be submitted to BD for approval:

- Details of the exploration, locations and method statements of the proposed exploration holes, trial pits, trenches, field tests, or instrumentations relative to WR structures within or outside the development site.

- The depth of proposed boreholes, trial pits, or trenches.

- If boreholes are to be sunk to a depth of 3 m from the highest point of the WR underground structures, method statement for checking verticality of the holes within a distance of 10 m on plan from any point of the railway structures is required.

- The vertical and horizontal pressure on any WR structure generated by ground investigation works such as field tests, plate loading test, pressure meter test, packer test or any other operation shall not exceed 20 kPa.

- Artificial shocks generated either by the detonation of explosives or a mechanical blow at ground surface or at depth within a drill hole shall not exceed 25 mm/sec peak particle velocity measured at any point on the surface of the WR structure.

- Percussion drilling, hammer drilling or any other site operation, which can induce prolonged vibration, shall not exceed 15mm/sec peak particle velocity measured at any point on the surface of the WR structure.

C.	Railway At-grade Section

- Utility trenches require excavation within the Railway Protection Area shall not be carried out without the approval of the Government in consultation with KCRC.  When utility works are to be carried out across the WR track at ground level, approval must be obtained from KCRC.

- Installation of pipes, drainage facilities, or other similar engineering works to be carried out within the Railway Protection Areas shall obtain Government’s prior approval in consultation with KCRC.

/3. …..

- Piling works will not be permitted within 7 m measured from the centreline of the nearest existing track without prior special railway arrangements.

- Ground vibration measured on the overhead line mast, or signalling and telecommunication furniture shall not exceed 10mm/sec and the amplitudes shall not exceed 80 µm.

D.	Railway Elevated Structures

1.	The erection of scaffolding, advertising signs or installation of any projections at a level greater than 1 m above the tracks shall not be allowed from any structure located within 6 m of the track, without the consent of the Government.  When the proposed works are permitted to be carried out to the structure located within the 6 m limit, effective measures shall be provided to protect the railway structures.  According to individual circumstances it may be necessary for the works to be conducted outside KCR hours of operation.  All protective works shall be subjected to special approval of Government.

2.	When cables and pipes, etc are to cross above or below WR elevated structures, the Utility Undertaking company shall submit to the Government details of the cables, pipes and the method of construction and seek special approval before work can commence.  When cables and pipes, etc are to pass over the railway, the provision of paragraph D.1 shall apply.

Operation of Stationary Lifting Appliance

- When operation of stationary lifting appliance i.e. tower crane, hoist, piling/drilling rigs etc. is adjacent to WR tracks and elevated structures, effective protection measures must be provided.  The operation time for the appliance may be limited to outside WR’s operating hours and shall vary according to individual circumstances.  All protective works are subject to special Government approval.  In addition the Government may impose conditions on the operation and positioning of the appliance if in its opinion such operation and positioning may endanger the safe operation of the WR.

- The hoisting appliance and hoisted materials should under no circumstances encroach the 3 m air space of the Land Reserve of the Railway or its overhead line equipment and in the event of collapse it should avoid damaging the overhead line equipment and structures.

- The swing movement of the jib of lifting appliance shall not encroach within 6 m of the WR tracks and its elevated structure on plan except with special Government approval.

- Where a mobile lifting appliance (e.g. cranes, excavator etc.) operates within 6 m on plan of WR structures and any part of the appliance higher than the WR track level, the provisions of paragraphs D.3, 4 & 5 above shall apply.

/E. …..

E. 	Maintenance and other Operations

Maintenance of Road Lamp Standards

- When road lighting maintenance vehicles are to be used to service road lamp standards adjacent to WR tracks, the working equipment shall not be positioned closer to the tracks than the nearest part of the road lamp standard where the height of the lamp standard is greater than the height of the tracks.  If the working equipment is to be operated at a level higher than 1 m above the tracks and closer than 6 m from the track, provisions of paragraph D.1 shall apply.

- Storage of Material

- No materials, containers, etc. shall be stored within 6 m on plan from the edge of WR structures/tracks and vent shaft openings except with special agreement of Government.

- No dangerous goods or inflammable materials shall be stored within 6 m on plan from the edge of WR structures/tracks except with special Government approval.

Demolition Works

- Where demolition and removal works for any structure which includes scaffolding, advertisement signs, container offices and buildings are proposed above or adjacent to WR above ground structures, such as entrances, vent shafts, distribution substations, traction power substations, plant rooms, elevated railway structures and at-grade track sections, effective protection measures for these railway structures shall be provided.  Demolition work proposal with the protection measures shall be submitted to BD for approval and obtain consent prior to commencing work.

Ref. :	BD GP/BORD/67

BD R&L/29K/99 Pt III

(8/2003)