| Buildings Department | Practice Note for Authorized Persons and Registered Structural Engineers | 279 |
| --- | --- | --- |

#### Kowloon-Canton Railway Corporation Railway Protection

Railways Ordinance

#### Buildings Ordinance Scheduled Area No. 3

To safeguard the safety and stability of the railway structures of the Kowloon-Canton Railway Corporation (KCRC), a “protection boundary” for the KCRC railways has been determined.  The areas within the protection boundary are commonly known as the railway protection areas.  In addition, the railway protection areas of the West Rail and East Rail Extensions – Hung Hom to Tsim Sha Tsui are designated in Scheduled Area No. 3 in the Fifth Schedule to the Buildings Ordinance.  The KCRC railway protection areas in Scheduled Area No. 3 are: -

- West Rail

the areas delineated on the plans numbered KCR/WR/RP100 to 120 (Rev. 1), KCR/WR/RP121 (Rev. 2) and KCR/WR/RP/122 to 145 (Rev. 1) which were signed by the Secretary for Housing, Planning and Lands on 6 June 2003 and deposited in the Land Registry; and

- East Rail Extensions - Hung Hom to Tsim Sha Tsui

the areas delineated on the plans numbered KCR/ERE/TSTE/RP/100, KCR/ERE/TSTE/RP/101 and KCR/ERE/TSTE/RP/111 which were signed by the Secretary for Housing, Planning and Lands on 1 June 2004 and deposited in the Land Registry.

Copies of these plans are available for inspection in the Buildings Department (BD) and the KCRC Railway Protection Unit.

2.		As a general guide, the protection boundary is about 30 m outside the outer surface of the railway structures but it encompasses the whole of any lot where any part lies within the 30 m distance.  At the railway stations, the area enclosed by the boundary is more extensive.

--------	3.		The guidelines at Appendix A apply to all building works (including ground investigation works and underground drainage works) to be carried out in the railway protection areas of the above KCRC railways.  Ground investigation works and underground drainage works in Scheduled Area No. 3 are subject to additional control as stated in paragraph 9 below.

/Building …..

- 2 -

Building Works

4.		Plans submitted to BD for proposed building works within the protection boundary are circulated to the KCRC for its comment under the centralized processing arrangements.  Railways Ordinance section 27 will be invoked to require the incorporation of any necessary measures to protect the railways.

5.		APs and RSEs are required to monitor any movement and vibration on the railway structures when required by the KCRC.  Upon request by the KCRC, APs and RSEs should submit a copy of the monitoring record to them.  APs and RSEs are required to inform the KCRC direct of the commencement of any building works within the protection boundary to enable them to plan an appropriate monitoring programme.  If any adverse situation becomes apparent, the KCRC will alert the interested parties.

6.		Any other necessary monitoring within the building site (e.g. monitoring of piezometric change) will be carried out by the AP/RSE of the site, who should maintain regular contact with the KCRC and keep each other informed of the monitoring records as necessary.

7.		The Secretary for the Environment, Transport and Works may in writing, under the provisions of Railways Ordinance section 24, authorize an employee of the KCRC to enter any building site within the protection boundary for the purpose of monitoring construction.

Building Opening adjacent to a Vent Shaft

8.		There are certain restraints on the design of and making alteration to properties in close proximity to a railway vent shaft to minimise the possibility of contamination by fire or smoke.  In this connection, APs and RSEs are advised that any opening such as an openable or fixed window, doorway, building ventilation system intake or exhaust and the like in any building shall be located not closer than 5 m to the opening of any railway vent shaft, irrespective of whether such vent shaft is free-standing or is accommodated in a building.  This distance may be reduced to 2.5 m, if the exhaust air from the railway vent shaft is directed away from and is not likely to affect the opening by natural convection.

Ground Investigation Works and Underground Drainage Works

9.		Ground investigation works within Scheduled Area No. 3 require prior approval and consent.  Plans prescribed under Building (Administration) Regulation 8(1)(l) should be submitted and the proposals should follow the guidelines at Appendix A.  Application for concurrent processing of approval and consent in respect of ground investigation works should follow PNAP 225.  Underground drainage works in Scheduled Area No. 3 are also subject to the full provisions of the Buildings Ordinance.

--------

/Railway …..

- 3 -

Railway Protection of Ma On Shan Rail

--------

10.		The guidelines at Appendix A also apply, where appropriate, to all building works to be carried out in the vicinity of the railway structures of Ma On Shan Rail (also known as East Rail Extensions - Tai Wai to Wu Kai Sha).  As a general guide, BD will circulate those building plans which involve building works within 30 m outside the outer surface of the railway structures to the KCRC for its comments on any necessary measures to protect the railway structures under the centralized processing arrangements.  Information on the protection boundary of the railway is available for viewing by AP/RSE at the Building Information Centre of BD.  It may also be obtained direct from the KCRC Railway Protection Unit.

(Marco M H WU)

Building Authority

Ref. :	BD GP/BORD/67

BD Rail/29K/99 Pt III

BD Rail/29M/03

First issue  August 2003 (AD/NB1)

This revision  March 2005 (AD/NB1) (Generally revised)

Index under :	Kowloon-Canton Railway Corporation Railway Protection

Scheduled Area No. 3

Appendix A

(PNAP 279)

Technical Notes for Guidance in Assessing

the Effects of Civil Engineering Construction / Building Development on

the Structures and Operations of

Kowloon-Canton Railway Corporation (KCRC) West Rail and

East Rail Extensions – Hung Hom to Tsim Sha Tsui and Tai Wai to Wu Kai Sha

A.	General

All proposals for new engineering works and developments (permanent or temporary) including public utilities located within the railway protection areas -shall be subjected to the special scrutiny of the Government prior to giving approval to any plans and consent for commencing construction works.  Every proposal shall be assessed individually on its impact on existing railway and related structures including all plants and fixtures necessary for the safe operation of the railway and subjected to satisfactory compliance with the following Technical Requirements.

- Railway Underground Structures

Site Formation /Foundation Works

Where any proposed  site formation, or foundation, or bulk excavation works for basement is located above or adjacent to the railway underground structures including cooling mains, the effects of such work shall not exceed the following limits:

- Vertical or horizontal pressure change caused either by dewatering, grouting, back filling, deep excavation, or loads transmitting from adjacent foundations or the combination of these works shall not exceed 20 kPa.

- Differential movement resulting from the works shall not produce accumulative distortion in any railway structure including the plinth and track in excess of 1 in 1000 in any plane or a total movement in any railway structure including the plinth or track exceeding 15mm in any plane whichever is smaller.

- The peak particle velocity induced to any railway structure as a result of permitted blasting work shall not exceed 25mm/sec.

- The peak particle velocity induced to any railway structure due to the driving or withdrawing of piles or any similar operation shall generally not exceed 15mm/sec, depending on the site conditions, and the duration and frequency of the activities.

/5. …..

- No pile, foundation of any kind, borehole or well, soil nails, horizontal drains, rock bolts/dowels or other geotechnical installations shall be driven or constructed within a distance of 3 m from any point of the underground railway structures.

- Any part of a ground anchor, if permitted, shall be installed more than 3 m away from any part of a railway structure, and the centroid of the fixed length of the anchor shall be located away from any tunnel installation at more than twice the fixed length.

Ground Investigation Works

When ground investigation is proposed within Scheduled Area No.3, a proposal with the following details and requirements shall be submitted to BD for approval:

- Details of the exploration, locations and method statements of the proposed exploration holes, trial pits, trenches, field tests, or instrumentations relative to railway structures within or outside the development site.

- The depth of proposed boreholes, trial pits, or trenches.

- If boreholes are to be sunk to a depth of 3 m from the highest point of the railway underground structures, method statement for checking verticality of the holes within a distance of 10 m on plan from any point of the railway structures is required.

- The vertical and horizontal pressure on any railway structure generated by ground investigation works such as field tests, plate loading test, pressure meter test, packer test or any other operation shall not exceed 20 kPa.

- Artificial shocks generated either by the detonation of explosives or a mechanical blow at ground surface or at depth within a drill hole shall not exceed 25 mm/sec peak particle velocity measured at any point on the surface of the railway structure.

- Percussion drilling, hammer drilling or any other site operation, which can induce prolonged vibration, shall not exceed 15mm/sec peak particle velocity measured at any point on the surface of the railway structure.

C.	Railway At-grade Section

- Utility trenches require excavation within the railway protection area shall not be carried out without the approval of the Government in consultation with the KCRC.  When utility works are to be carried out across the railway track at ground level, approval must be obtained from KCRC.

- Installation of pipes, drainage facilities, or other similar engineering works to be carried out within the railway protection areas shall obtain Government’s prior approval in consultation with the KCRC.

/3. …..

- Piling works will not be permitted within 7 m measured from the centreline of the nearest existing track without prior special railway arrangements.

- Ground vibration measured on the overhead line mast, or signalling and telecommunication furniture shall not exceed 10mm/sec and the amplitudes shall not exceed 80 µm.

- The requirements of paragraphs B.2 to B.6 shall also apply to at-grade structures.

D.	Railway Elevated Structures

1.	The erection of scaffolding, advertising signs or installation of any projections above the tracks shall not be allowed within 6 m on plan of any point of the track, without the consent of the Government.  When the proposed works are permitted to be carried out to the structure located within the 6 m limit, effective measures shall be provided to protect the railway structures.  According to individual circumstances it may be necessary for the works to be conducted outside KCR hours of operation.  All protective works shall be subjected to special approval of Government.

- When cables and pipes, etc are to cross above or below railway elevated structures, the Utility Undertaking company shall submit to the Government details of the cables, pipes and the method of construction and seek special approval before work can commence.  When cables and pipes, etc are to pass over the railway, the provision of paragraph D.1 shall apply.

- The requirements of paragraphs B.2 to B.6 shall also apply to elevated structures.

E. 	Maintenance and other Operations

Maintenance of Road Lamp Standards

- When road lighting maintenance vehicles are to be used to service road lamp standards adjacent to railway tracks, the working equipment shall not be positioned closer to the tracks than the nearest part of the road lamp standard where the height of the lamp standard is greater than the height of the tracks.  If the working equipment is to be operated at a level higher than 1 m above the tracks and closer than 6 m from the track, provisions of paragraph D.1 shall apply.

- Storage of Material

- No materials, containers, etc. shall be stored within 6 m on plan from the edge of railway structures/tracks and vent shaft openings except with special agreement of Government.

/3. …..

- No dangerous goods or inflammable materials shall be stored within 6 m on plan from the edge of railway structures/tracks except with special Government approval.

Demolition Works

- Where demolition and removal works for any structure which includes scaffolding, advertisement signs, container offices and buildings are proposed above or adjacent to railway above ground structures, such as entrances, vent shafts, distribution substations, traction power substations, plant rooms, elevated railway structures and at-grade track sections, effective protection measures for these railway structures shall be provided.  Demolition work proposal with the protection measures shall be submitted to BD for approval and obtain consent prior to commencing work.

Operation of Stationary Lifting Appliance

5.	When operation of stationary lifting appliance i.e. tower crane, hoist, piling/drilling rigs etc. is adjacent to railway tracks and elevated structures, effective protection measures must be provided.  The operation time for the appliance may be limited to outside railway operating hours and shall vary according to individual circumstances.  All protective works are subject to special Government approval.  In addition the Government may impose conditions on the operation and positioning of the appliance if in its opinion such operation and positioning may endanger the safe operation of the railways.

6.	The hoisting appliance and hoisted materials should under no circumstances encroach the 3 m air space of the Land Reserve of the Railway or its overhead line equipment and in the event of collapse it should avoid damaging the overhead line equipment and structures.

7.	The swing movement of the jib of lifting appliance shall not encroach within 6 m of the railway tracks and its elevated structure on plan except with special Government approval.

8.	Where a mobile lifting appliance (e.g. cranes, excavator etc.) operates within 6 m on plan of railway structures and any part of the appliance higher than the railway track level, the provisions of paragraphs E.5, 6 & 7 above shall apply.

Ref. :	BD GP/BORD/67

BD Rail/29K/99 Pt III

BD Rail/29M/03

(Rev. 3/2005)