| ### Buildings Department | Practice Note for Authorized Persons, Registered Structural Engineers and Registered Geotechnical Engineers | APP-147 |
| --- | --- | --- |

Minor Works Control System

Implementation

The Buildings Ordinance (“BO”) has been amended by the Buildings (Amendment) Ordinance 2008 and a set of new regulations, the Building (Minor Works) Regulation [“B(MW)R”] has been made to introduce a new system, the Minor Works Control System (“MWCS”) to control the carrying out of minor works.  Apart from the old system under which prior approval and consent of the Building Authority (“BA”) have to be sought for carrying out building works, a set of prescribed requirements that are simpler than the old system has been added to the BO by the above Amendment Ordinance and new regulations.  The MWCS comes into operation on 31 December 2010.

Classification of Minor Works

2.	A new category of building works, namely “minor works”, and a new register of “registered minor works contractors” (“RMWCs”) for carrying out such minor works, have been introduced under the BO.  In respect of minor works, the requirement to seek BA’s prior approval for building plans and consent to commence the works can be dispensed with.  Minor works are classified into three classes according to their nature, scale and complexity as well as the risk to safety they pose:

- Class I minor works are relatively more complicated minor works, e.g. installation of internal staircases connecting two floors;

- Class II minor works are comparatively less complex works, e.g. repair of external walls; and

- Class III minor works cover small-scale minor works, mostly carried out in household settings, e.g. erection of supporting frames for air-conditioners.

3.	The various items of minor works are described in Schedule 1 of the B(MW)R.

/The …..

The Simplified Requirements

4.	As an alternative to obtaining prior approval and consent from the BA, minor works can be carried out under the simplified requirements of the MWCS, the requirements of which are tabulated as follows for reference:

| Minor works involved Requirements | Class I | Class II | Class III |
| --- | --- | --- | --- |
| Appointment (of the Appointed Person) | - PBP to design and supervise the works; and - PRC to carry out the works | - PRC to prepare and submit the plans and documents, to supervise and carry out the works |  |
| Submission | AP | PRC | PRC |
|  | - Notice of Commencement of Works; and - Certificate of Completion of Works |  | - Notice and Certificate of Completion of Works |

PBP	:	Prescribed Building Professionals – an Authorized Person (AP), [if necessary, and a Registered Structural Engineer (RSE) and/or a Registered Geotechnical Engineer (RGE)]

PRC	:	Prescribed Registered Contractor – Registered General Building Contractor (RGBC), Registered Specialist Contractor (RSC) or Registered Minor Works Contractor (RMWC)

5.	The workflow of the simplified requirements and the documents required to be submitted are summarized in Appendix A.

______

6.	Some requirements and general guidance to streamline and expedite the processing of submissions are provided in Appendix B.  The specified forms and standard forms for use under the simplified requirements are listed in Appendix C.

______

______

General Reminders

7.	Under section 55 of the B(MW)R, the AP appointed in respect of any Class I minor works is required to provide a copy of the prescribed plans, details of the works and supervision plan (as required) submitted under the simplified requirements to the PRC for their keeping on site.  The AP shall also remind the PRC to keep a copy of the Buildings Department’s acknowledgement letter for the “Notice of Commencement of Works” on site for inspection by officers of the Buildings Department (BD).

8.	If the minor works involve common parts of a building, the AP should remind “the person who arranged for the works to be carried out” to pay attention to the civil liabilities under the deed of mutual covenant of the building.  The AP should also notify the management office concerned, the Incorporated Owners or the related owner(s) about the minor works to be carried out.

/9.  …..

9.	The AP should remind the PRC to take appropriate measures to ensure the proper disposal of construction waste.

10.	Where a supervision plan is required, the PBP and PRC are reminded to comply with the requirements under the Technical Memorandum for Supervision Plans (TM) and the Code of Practice for Site Supervision.

11.	When the AP, RSE or RGE ceases to be appointed or nominated as the PBP in respect of any Class I minor works, he should submit the standard form to notify the BA within 7 days after the date of the cessation.  When the PRC ceases to be appointed in respect of any Class I minor works, he should deliver the specified form to the AP within 7 days after the date of the cessation.  The AP should submit the form to the BA within 7 days after the date of receipt of such form.

Minor Works Relating to Signboards

12.	In order to facilitate the distinction of the signboards erected or altered under the simplified requirements from those unauthorized signboards, it is recommended that the minor works submission numbers be displayed on the signboards erected or altered under the simplified requirements.  Standards and guidelines on display of these numbers are provided in Appendix D.  Similar arrangement is also recommended for those signboards which have been erected or altered with prior approval and consent from the BA as set out in PNAP APP-126.

______

Minor Works Relating to Radio Base Station, Antenna or Transceiver on Roof

13.	For minor works relating to radio base station, antenna or transceiver on roof, the AP and RSE should make reference to Appendix E for more details.

______

Inspection and Certification of Prescribed Building or Building Works (commonly known as “Household Minor Works Validation Scheme”)

14.	It is recognized that some unauthorized minor amenity structures may have been erected to meet practical household needs before the full implementation of the MWCS on 31 December 2010.  They include metal supporting frames for air-conditioning units or water cooling towers, drying racks and small canopies.  They are known as the prescribed building or building works (PB/BW), which are described in Schedule 3 of the B(MW)R.  Under the new section 39C of the BO and section 62 of the B(MW)R, an AP, RSE, RGBC or RMWC1 can be appointed to inspect and certify that the PB/BW are structurally safe and comply with the BO2.  Upon certification, no enforcement action will be taken against the three types of minor unauthorized structures unless their safety conditions change.  Alteration and/or strengthening works may be carried out before such minor structures can be certified.

/15.    …..

_________________________________

Note 1	RMWCs who are registered for type A minor works, type E minor works or item 3.25, 3.27, 3.28, 3.29, 3.34, 3.35, 3.36, 3.37 or 3.38 of Class III minor works

Note 2	Except BO section 14(1) and Building (Administration) Regulation 25

15.	A flow chart showing the steps of processing the submissions and the documents required to be submitted under the “Household Minor Works Validation Scheme” are provided in Appendix A.

______

No Referral of Plans

16.	Unlike the “centralized processing of building plans”, the documents submitted to the BD for the carrying out of minor works under the simplified requirements and for the inspection and certification of PB/BW will not be referred to other government departments.  If the proposed minor works involves the jurisdictions of other government departments, the AP or PRC should consult and obtain necessary approval or consent from the relevant government departments accordingly.

No Submission Fee Required

17.	All submissions under the simplified requirements and for the inspection and certification of PB/BW are free of charge.

Technical Guidelines on Minor Works Control System

18.	The “Technical Guidelines on Minor Works Control System” has been published by the BD, providing detailed guidance on compliance with the simplified requirements to the registered contractors and practitioners in the building industry.  These Guidelines are available at the BD’s website (http://www.bd.gov.hk) under the “Codes of Practice and Design Manuals” page of the “Publication” section.

19.	A similar practice note is issued to all Registered Contractors.

( AU Choi-kai )

Building Authority

Ref. : BD GP/BORD/113

First issue	December 2010 (AD/Sup)